Bài 6. ESP32 gửi nhận dữ liệu qua MQTT

Mục đích:

Lập trình cho ESP32 gửi nhận dữ liệu qua MQTT

Sử dụng công cụ mô phỏng wokwi https://wokwi.com/

Hoạt động của chương trình:

a) Gửi dữ liệu thu thập từ sensor (nhiệt độ, độ ẩm) qua MQTT broker

b) Nhận dữ liệu qua MQTT broker, điều khiển LED on/off tương ứng

Sử dụng MQTT broker có sẵn: broker.emqx.io

Yêu cầu nộp bài:

Viết báo cáo mô tả công việc thực hiện gồm:

- Code của chương trình

- Link bài mô phỏng trên wokwi

- Chụp ảnh màn hình kết quả thực hiện mô phỏng.

Submit: file báo cáo (kèm link wokwi bài mô phỏng)

Hướng dẫn tham khảo:

https://github.com/knolleary/pubsubclient/blob/master/examples/mqtt_esp8266/mqtt_esp8266.ino

Code tham khảo

// From https://randomnerdtutorials.com/esp32-mqtt-publish-subscribe-arduino-ide

#include <WiFi.h>

#include "PubSubClient.h"

#include "ArduinoJson.h"

const char* ssid = "Wokwi-GUEST";

const char* password = "";

const char* mqttServer = "broker.emqx.io";

int port = 1883;

String stMac;

char mac[50];

char clientId[50];

WiFiClient espClient;

PubSubClient client(espClient);

const int ledPin = 32;

const int ledPin2 = 25;

void setup() {

Serial.begin(115200);

randomSeed(analogRead(0));

delay(10);

Serial.println();

Serial.print("Connecting to ");

Serial.println(ssid);

wifiConnect();

Serial.println("");

Serial.println("WiFi connected");

Serial.println("IP address: ");

Serial.println(WiFi.localIP());

Serial.println(WiFi.macAddress());

stMac = WiFi.macAddress();

stMac.replace(":", "_");

Serial.println(stMac);

client.setServer(mqttServer, port);

client.setCallback(callback);

pinMode(ledPin, OUTPUT);

pinMode(ledPin2, OUTPUT);

}

void wifiConnect() {

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

}

void mqttReconnect() {

while (!client.connected()) {

Serial.print("Attempting MQTT connection...");

sprintf(clientId, "clientId_%s", stMac.c_str());

if (client.connect(clientId)) {

Serial.print(clientId);

Serial.println(" connected");

client.subscribe("esp32/led");

} else {

Serial.print("failed, rc=");

Serial.print(client.state());

Serial.println(" try again in 5 seconds");

delay(5000);

}

}

}

void callback(char* topic, byte* message, unsigned int length) {

Serial.print("Message arrived on topic: ");

Serial.print(topic);

Serial.print(". Message: ");

String stMessage;

for (int i = 0; i < length; i++) {

Serial.print((char)message[i]);

stMessage += (char)message[i];

}

Serial.println();

if (String(topic) == "esp32/led") {

StaticJsonDocument<200> doc;

deserializeJson(doc, stMessage);

int val1 = doc["led1"];

int val2 = doc["led2"];

Serial.println(val2);

Serial.print("Changing output to ");

if(val1 == 1){

Serial.println("on");

digitalWrite(ledPin, HIGH);

}

else if(val1 == 0){

Serial.println("off");

digitalWrite(ledPin, LOW);

}

if(val2 == 1){

digitalWrite(ledPin2, HIGH);

}

else{

digitalWrite(ledPin2, LOW);

}

}

}

void loop() {

delay(10);

float tempVal = 35;

float humidVal = 78;

String data = "{\"ClientID\":" + String(clientId) + ",\"Temp\": " + String(tempVal)+ ", \"Humid\": " + String(humidVal) + "}";

if (!client.connected()) {

mqttReconnect();

}

else{

Serial.println("Sending data to broker ...");

client.publish("esp32/out", data.c_str());

delay(5000);

}

client.loop();

}